TL604 vs. DG419

| TL604 pin # |  | DG419 pin # |
| --- | --- | --- |
| 1 | Connects to: | 3 |
| 2 | Connects to: | 6 |
| 3 | Connects to: | 8 |
| 4 | Connects to: | 2 |
| 5 | Connects to: | 7 |
| 6 | Connects to: | 1 |
| 7 | Connects to: | N/A |
| 8 | Connects to: | 4 |

Pin 5 of the DG419 must see +5VDC for the chip to work properly.  This can be done with a voltage divider network connected to the +15VDC supply:

1.  Connect a 2.2k resistor from the +15 VDC supply or pin 4 of the DG419

2.  Connect a 1k resistor in series with this 2.2k resistor to ground.

3.  Connect the 2.2k/1k node to pin 5 of the DG419.

For older 400RB and 800RBs only:

Another way around replacing the TL604, is to remove the chip and add a jumper wire where pin 3 is on the board, over to pin 6/7.  This allows the signal to pass through the rest of the preamp.  However, this permanently disables the footswitch function.  We do not recommend doing this with other GK products that use this chip.  Doing so, will eliminate a lot of features such as channel and effects switching!!!