Line 6 DL4 power supply

does any of them work with batteries?..
I used to work as repairman for the Scandinavian Line-6 distributers, so I think I can help...due to signed non-disclosure agreements, I cant send you copies of the schematics, but i can guide you through.

So follow me now...
...first thing to do. Plug a mono plug into the mono input, press footswitches 1 and 4 down and apply power. Don’t let go of the switches for at least 5 seconds. if nothing happens, its time to open up.

 From the AC-in, power goes through a 3,6v zener diode ( D16 ) then on through D11. the 3,6v zener are known to fail, but does not affect battery operation.

After  D11, power meets up with the battery in... next comes U14, a little 8-pin ( SMD ) MAX887. this one takes things down to 3,3volts ( measure pin7/ signal ground ). If you have 3,3volt on pin7, go on to U20.

U20 is a MAX660 which is a voltage doubler.  Check the DC voltage on pin1 ( or 8, they are tied together ). if you have 6,6volt here, next in line is a tiny thing called LM3480 ( U21) this one will even out things for the digital domain to 5 Volt...if you have not found fault yet, and reset does not work, don’t bother.

..so the most likely suspects are 
1: 3,6v Zener ( D16 )
2: MAX887 ( U14 )
3: MAX660 ( U20 )
4: LM3480 ( U21 )